BLC-015 .User Guide

Version Eng-1.00

- 2. Spec

| Item | SPEC | Remarks |
| --- | --- | --- |
| Name | VIRDI BLC-015 |  |
| Communiation | ① RS485 ② Wiegand |  |
| Power | DC 12V |  |
| In / Out | -. Door access -. RS485, Wiegand -. Battery In / Out |  |

- 3. Configuration

- 3.1. Front

SW1

SW5

SW4

SW3

SW2

COM

WIN0

WIN1

GND

485A

485B

EXIT

GND

N.C.

N.O.

LED

12V IN

GND

12V OUT

GND

BAT(+)

BAT(-)

- 4. Setting

- 4.1. Controller ID

Dip Switch 1~2

| No | Controller ID | Dip SW1 | Dip SW2 |
| --- | --- | --- | --- |
| 1 | 0 | OFF | OFF |
| 2 | 1 | OFF | ON |
| 3 | 2 | ON | OFF |
| 4 | 3 | ON | ON |

Use SW1 and SW@ to set up controller ID (01 ~ 04)

4.3 Set time of opening a door

Dip Switch 3~5

| No | Open Seconds | Dip SW3 | Dip SW4 | Dip SW5 |
| --- | --- | --- | --- | --- |
| 1 | 1 | OFF | OFF | OFF |
| 2 | 2 | OFF | OFF | ON |
| 3 | 3 | OFF | ON | OFF |
| 4 | 4 | OFF | ON | ON |
| 5 | 5 | ON | OFF | OFF |
| 6 | 6 | ON | OFF | ON |
| 7 | 7 | ON | ON | OFF |
| 8 | 8 | ON | ON | ON |

Use SW3 and SW4 to set up opening door time

- 5. Install and operation

- 5.1. Install VIRDI BLC015

1)  Connect between Virdi terminal and BLC015 via RS 485 or Wiegand

2)  Connect a lock to BLC-015

3)  Set controller ID and opening door time

3)  Power on a terminal and BLC-015

4)  Check POWER LED on

- 5. Connection

- 5.1. Lock

1)  Power

-  Adapter power : 12V IN,GND

-  Terminal : 12V OUT,GND

-  Battery power : BAT(+),BAT(-)

2)  Communication

-  RS-485 : Connect 485A and 485B

-  Wiegand : Connect  WIN0 and WIN1

3)  Lock

-  Connect COM and GND

-  Exit Button : Connect EXIT and GND

-  Lock의 	(+)  	12V OUT

(-) 	 	NC (NO)