HD DVR User Manual

HD Hybrid System

Contents

1	Starting	4

Attentions	4

1.1	Version Management	6

1.2	Products Lineup	7

2	DVR Main	8

2.1	Screen Description	8

2.2	Login	9

2.3	Camera/Sensor/Relay	10

2.4	PTZ Use	12

2.5	Live Audio	13

2.6	Screen color Adjustment	13

2.7	Rest function description	14

3	DVR Set up	17

3.1	System Set up	17

3.2	Camera Set up	20

3.3	Recording  setup	21

3.4	Motion setup	25

3.5	Sensor setup	29

3.6	Relay setup	30

3.7	Screen setup	31

3.8	Network setup	33

3.9	User setup	36

4	DVR Search	38

4.1	Screen description	38

4.2	How to play image 	39

4.3	Split screen	40

4.4	Panorama search	40

4.5	Image enlargement	41

4.6	Bookmark	42

4.7	Image save	43

4.8	Smart search	46

4.9	Image print	47

4.10	Image color adjustment	48

4.11	Selection for data image	49

Starting

This user manual is the basic one for installation and operation of HD Hybrid Series. We recommend users who are using this product for the first time or not accustomed to read this manual before installation.

This user manual could be changed for upgrading products specs and functions.

- Attentions

Please use this product after recognition of the below contents for security and to reduce trouble.

- Don’t shock to the product when moving or installing.

- Don’t set up the product at the vibrating place. It could be caused trouble.

- Don’t move the product when operating it.

- Don’t pull the plug out when operating the product.

- Please clean the product with dried towel after turning off the power.

- Don’t pull and plug out the electric power and touch the electric plug with wet hand.

- Don’t put the heavy things on the power code. If using damaged power code, it could cause burning or electric shock.

- Don’t open the case due to damage of electric shock inside.

- Please set up the product at the flat place and keep it at proper temperature.  It could cause a fire in case of setting up at the closed space without circulation of air.

- Please contact to the seller after turning off the power if it smells or smokes.

- Please keep away the product from the place exposed to direct sunlight or beside of heater.

- Don’t put the burnable materials beside the product. It can cause a fire.

- Please contact to the seller when upgrading and installing more HDD.

- Don’t use the product with the other usage after taking apart and converting the product at discretion.

- Don’t install the programs such as games and messenger except for the allowed S/W..

Version Management

| - Date | - Version | - Contents |
| --- | --- | --- |
| - 2010-07-31 | - Rev 1.0 | - First version of user’s manual |

Products Lineup

| Lineup | Content |
| --- | --- |
|  | System |
|  | Power cable and Key |
|  | Product CD (Program, Manual, S/W included) |
|  | Keyboard, Mouse |

DVR Main

Screen Description

(1)

(4)

(5)

(2)

(3)

(6)

<Picture 2.1>

| No. | Description |
| --- | --- |
| (1) | Main Surveillance Screen |
| (2) | Screen division window. It can divide as 1,4,6,9,10,13,16,20 channel. It supports full screen, split screen, page switch, auto page switch. |
| (3) | It basically indicates time. And it supports live audio, Screen color adjustment, system information and backup. |
| (4) | Screen Login, Setup, search, PTZ, |
| (5) | It supports an installed camera information, sensor, relay information, and controls PTZ. |
| (6) | System Exit |

Login

In order to use system, please login here.

<Picture 2.2>

Basic user ID : admin , PW : 1111 .

If you don’t certify as user, you can’t use other functions of main screen and usable functions are limited by user’s right.

User’s exact rights are in user’s setup of “Setup”.

Camera/Sensor/Relay

Camera Information

<Picture 2.3.1>

It shows information on HD camera resolution and analog camera.

<Picture 2.3.1> shows information on cameras connected to present system, and outputting camera description and resolution in HD CCTV, and outputting camera information in Analog CCTV.

[P] appears in PTZ camera, [A] appears if audio is set up, in case of Video Loss, camera icon appears to red color.

The followings are the present supported camera resolution.

| Items | Resolution(NTSC/PAL) |
| --- | --- |
| HD CCTV | 1280 x 720 60P(50) 1920 x 1080 30P(25) 1920 x 1080 60I(50) |
| Analog CCTV | 360 x 240(360 x 288) 720 x 240(720 x 288) 720 x 480(720 x 576) |

Sensor/Relay

(1)

(2)

<Picture 2.3.2>

Relay information connected with system expresses like <Picture 2.3.2>.

In case of relay, (1) indicates in <Picture 2.3.2>. Relay operates if user clicks a relay icon and color changes to Green..

In case of sensor, (2) indicates in <Picture 2.3.2>. Icon changes when using.

PTZ Use

<Picture 2.4>

In case of using PTZ in selected channel, if user selects Pantilt, <Picture 2.4> occurs.

PTZ could be controlled in <Picture 2.4>, it activates automatically if the camera supports Preset, Touring, Scan.

Please refer appendix for more details.

Live Audio

<

<Picture.2.5>

If you set up audio of the channel you choose, [A] appears on the channel at <Picture. 2.3.1>. If you want to hear live audio, choose a channel you want to hear and click the audio at <Picture 2.1>.

Please choose “Audio”, <Picture 2.5> screen will come out and then press “Play” button, can hear audio.

| Items | Description |
| --- | --- |
| Apply all | It applies to the all channels set up. |
| Default | Default set by in the system applies to volume. |
| Save | It saves the current volume set by in the channel. |

Screen color Adjustment

(1)

<Picture 2.6>

If you choose “color” button of <Picture 2.1>, <Picture 2.6>

appears. Please adjust screen color through (1) of <Picture 2.6>

You can adjust “Brightness” , ”Contrast”, ”Saturation”,  ”Hue” from the left side.

| Items | Description |
| --- | --- |
| Apply all | It applies to the all channels set up. |
| Default | Default set by in the system applies to color. |
| Save | It saves the current volume set up by in the channel. |

Rest function description

<picture 2.7>

If you choose “other” in (3) of <Picture 2.7>, it supports additional functions as below.

Information

<Picture 2.7.1>

If you choose “Information” in <Picture 2.7>, it displays system information and installation software information like  <Picture 2.7.1>.

This information would be used as important one when system trouble happens.

Backup

(1)

(2)

(3)

(4)

(5)

<Picture 2.7.2>

If you choose “Backup” in <picture 2.7>, <picture 2.7.2> appears. You can backup after choosing the related channel.

| Items | Description |
| --- | --- |
| (1) | Choose a camera to backup. (You can choose several cameras.) |
| (2) | Put the starting time and ending time. |
| (3) | Select a device to save backed up information. Backed up information is saved under “C:\_Backup” and in case of inputting “Folder name”, “C:\_Backup\folder name” is created and saved. |
| (4) | It describes the process of backup. |
| (5) | Press “Backup start” Button. |

Please refer appendix for more details.

Log View

<그림 2.7.3>

If you choose “Logview” in <Picture 2.7>, <picture 2.7.3> appears.  Please search and output “System log”, “Event log”, “Network Log” .

CD Burn

(2)

(1)

(3)

<Picture 2.7.4>

If you choose “ CD Burn” in <picture 2.7>,  <picture 2.7.4> appears. You can copy backup information or image file to CD/DVD.

You examine device through in <picture 2.7.4> and add folder or file to copy through (2) and then CD/DVD is made by (3).

DVR Set up

You can set up system after choosing “Set up” <picture 2.1>(4)

System Set up

(1)

(6)

(5)

(4)

(3)

(2)

<Picture 3.1>

Basic set up of system is like <picture 3.1>. How to set up is as below.

| Items | Description |
| --- | --- |
| (1) | It describes that camera is using or not. Camera button is grey when it is not using. |
| (2) | Input the system’s name in “description”. ”Video type” divides with “NTSC/PAL”. |
| (3) | “Log use” logs when sensor works and you can check them through log search. Refer <2.7.3>. “Automatic Logout time set up” means that if user don’t use mouse within setting time, it automatically log outs. “Watchdog use”. System automatically judges as false and reboots if board or input of video occurs within set time. “Summer time use” - When time reversal occurs according to summer time, it backs up information as much as time reversal occurs to the folder designated. |
| (4) | This is a function that sets up how to notify when “Video Loss” occurs. There are “Beep notify”, “CMS notify”, E-mail notify”. |
| (5) | Automatic reboot and exit of system. |
| (6) | Schedule back up. |

Summer time Backup set up

(1)

<picture 3.1.1>

Image Data can be automatically backup if summer time is set up like <picture 3.1.1>(1).

Automatic reboot/Exit set up

(1)

(2)

<picture 3.1.2>

Automatic reboot and exit can be set up hour and minute after setting up days or weekdays like <picture 3.1.2>(1) and (2).

(1)

Schedule backup set up

(3)

(2)

(4)

<picture 3.1.3>

Schedule backup supports the following 2 kinds

Schedule back up : Video data for a fixed time in the <picture 3.1.3> (3) can be back up to the folder of (2).

Simultaneous backup : This system has some saving space and save the video data in there.

This space calls “Basket”, system uses hundreds or thousands of basket.

Simultaneous backup means that if video is saved in a basket and backup the basket in the (2) at the same time.

Camera Set up

(1)

(2)

(3)

(4)

<picture 3.2>

Camera set up is <picture 3.2>.

| Items | Description |
| --- | --- |
| (1) | Choose a camera to set up. |
| (2) | Input a camera name in “camera description”. ”Secret mode use” means that video save is in the process, but displays that video is not using on main surveillance screen. |
| (3) | If the chosen camera supports PTZ, check “Use” and input camera address and type. More detailed contents are in the appendix. |
| (4) | Please set up audio or not and save or not in the selected camera. |

Recording  setup

(3)

(2)

(4)

(1)

<picture 3.3>

You can set up recording setup for the system like <Picture 3.3>

| Items | Description |
| --- | --- |
| (1) | Select a camera to set up. The button “Apply all” below is used to apply recording setup to all cameras. (When you select HD camera, save setup applies to all HD cameras. In case of analog cameras, save setup applies to all analog cameras. |
| (2) | It sets the resolution and image quality of the cameras you select. |
| (3) | It sets up a holiday and the recording time when events occur. |
| (4) | It sets up recording schedule. |

Setup for input resolution

(2) of <Picture 3.3> sets up camera’s resolution for input and compression saving.

(1)

(2)

<Picture 3.3.1>

(1) in <Picture 3.3.1> is a screen to set up for HD Camera.

“Resolution” finds and shows the input resolution of cameras.

“Resolution for saving” can save as same as input resolution of camera and 1/4, 1/16 of input resolution of camera.

“saving speed” sets up fps for images.

“ Compression Type” sets up the size of compressed image with either CBR(constant bit rate) or VBR(variable bit rate).

“image quality” sets up image quality for images to save.

(2) in <Picture 3.3.1> is a screen for setup when you select analog cameras.

“Resolution” sets up the resolution for input.

“Saving speed” sets up fps for images.

Holiday setup

You can do “holiday setup” at (3) in <picture 3.3>.

(1)

<picture 3.3.2>

You can set up holiday inputting holiday and description at (1) of <picture 3.3.2> and clicking “Add” button.

Recording schedule for holiday is same as the recording schedule for Sunday.

Recording schedule setup

You can set up camera recording schedule at (4) of <picture 3.3>.

(1)

(3)

(2)

<picture 3.3.3>

Select day/time box below after selecting the type of saving at (1) of <picture 3.3.3>..

| items | description |
| --- | --- |
| (1) | “ No saving” : let you only watch images and doesn’t save the image. “constant saving” : saves images constantly. “motion saving” : saves images when it senses any motions in the image. “sensor saving” : saves images when sensor works. “sensor or motion” : saves images when sensor works or senses any motion on the screen. “normal & event” : usually saves 1fps and saves when it senses events(motion, sensor) Note: “Motion saving”, “Sensor saving” and “normal&event saving” save images for the period(seconds) set by at “ recording setup for events” of (3) in <picture 3.3> |
| (2) | “A” displays days and you can select all days and time by clicking |
| (3) | It displays time. |

Motion setup

(5)

(4)

(3)

(2)

(1)

<picture 3.4>

You can set up motion of the system on the screen like <picture 3.4>.

| items | Description |
| --- | --- |
| (1) | Select a camera to set up motion. The button “Apply all” below is used to apply recording setup to all cameras. (When you select HD camera, saving setup applies to all HD camera. In case of analog camera, saving setup applies all analog cameras. |
| (2) | It sets up the motion of camera you select. |
| (3) | It sets up “notify method” in case events occur. “full screen” : prints out images with full screen from the camera that event occurred. “Beep” : beeps when events occur. “screen printout” : prints out events when events occur. “display duration time” : sets up “Event notify” schedule(seconds). |
| (4) | It sets up “notify method” for remote place when events occur. “RMS notify” : notifies a remote place management program of the situation of events. “e-mail notify” : notifies you of events through e-mail. For “RMS” and “e-mail” setup, refer to appendix for network setup. “schedule setup” : sets up “notify schedule”. |
| (5) | It sets up relay operation in case events occur. |

Motion area setup

(4)

(3)

(2)

(1)

<picture 3.4.1>

You can set up motion on the screen like <picture 3.4.1> after clicking “ motion area setup” at (2) of <picture 3.4>.

| items | Description |
| --- | --- |
| (1) | Select a camera to set up motion area. |
| (2) | Select a motion sensing area. “Full area” : sets up all images as sensing area. “ Partial area” : sets up a image’s partial area as sensing area. You can designate 8 partial areas. |
| (3) | It able automatically when you select a motion sensing area as “partial area”. “Add” sets up an area dragging the mouse after selecting. “Select”: you can select square area after pressing this button when selecting one particular area among partial areas. “Delete” : deletes selected partial areas. “Delete all” : deletes all partial sensing area. |
| (4) | It sets up motion sensitivity of the selected sensing area. The lower “Motion sensitivity” and “sensitivity” are the better it senses. “Test” : tests motion sensitivity directly. It displays “sensed” if it senses motions. |

Schedule setup

(1)

(3)

(2)

(4)

<picture 3.4.2>

You can set up a notify schedule on the screen like <picture 3.4.2> after selecting “schedule setup” at (4) of <picture 3.4>.

| items | description |
| --- | --- |
| (1) | Select a type of events. “Motion” sets up a “notify schedule” when motions occur. “Sensor” sets up a “notify schedule” when sensor works. |
| (2) | Select a camera you will apply “notify schedule”. |
| (3) | It sets up a RMS notify schedule. You can set up by selecting “No operation”(purple color)“ and day/time box below. You can set up after selecting “operation”(green color) and day/time box below. |
| (4) | It sets up a Email notify schedule. You can set up by selecting “No operation”(purple color)“ and day/time box below. You can set up after selecting “operation”(green color) and day/time box below. |

Sensor setup

(1)

(2)

(3)

(4)

(5)

(6)

<picture 3.5>

You can set up sensor on the screen like <picture 3.5>.

| items | Description |
| --- | --- |
| (1) | Select a sensor number to set up. |
| (2) | Set up a name and sensor type. It supports two sensor types which are “Normal Open” and “Normal Close”. |
| (3) | It sets up how to operate and operation time when sensor events occur. “full screen” : prints out images with full screen from the camera connected to sensors “ “Beep” : beeps when sensor works“ “screen display” : displays events when sensor works. “display duration time” : sets up the time(seconds) to operate when sensor works. “ |
| (4) | It sets up how to notify remote places when sensor events occur. “RMS notify” : notifies a situation that sensor works to remote management program. “Email notify” : notifies a situation that sensor works through e-mail. For “RMS” and “e-mail” setup, “ refer to appendix for network setup Refer to <3.4.2> for schedule setup. |
| (5) | Select camera to be connected to selected sensor. |
| (6) | Select relay to be connected to selected sensor. |

Relay setup

(1)

(2)

(3)

<picture 3.6>

You can set up relay on the screen like <picture 3.6>.

| items | Description |
| --- | --- |
| (1) | Select relay number to set up. |
| (2) | Input name and operation time(seconds) of selected relay. |
| (3) | Set up working schedule for selected relay. Set up by clicking day/time box below after clicking “no operation”. Set up by clicking day/time box below after clicking “operation”. |

Screen setup

(1)

(2)

(3)

<picture 3.7>

You can set up system’s surveillance screen on the screen like <picture 3.7>.

| items | Description |
| --- | --- |
| (1) | Select a camera to set up surveillance screen. |
| (2) | Default screen -“Use full screen” prints out default screen with full screen. - -“1,4,6,9,10,13,16,20” supports split surveillance screen. Page auto switch - It sets up surveillance page auto switching time(second)- Information Display It sets up whether to display camera information on the surveillance screen or not. -“Font” sets up character’s font, color, size for camera information display- External monitor printout “External monitor printout setup” prints out surveillance screen on the other monitors.- |
| (3) | You can clear the cameras allocated to split screen using “Clear all”. If you drag cameras at (1) to channel screen (3), the cameras will be allocated to the channel you dragged to. <Note> If you don’t allocate cameras, it will save according to save schedule but doesn’t print out on the screen. |

External monitor printout setup

(2)

(1)

(3)

(4)

<picture 3.7.1>

You can set up external monitor printout on the screen like <picture 3.7.1>.

| Items | description |
| --- | --- |
| (1) | Select a camera to print out on external monitor. You can choose one or several cameras and the cameras you select appears green. |
| (2) | “Split” sets up external monitor printout as 1 split or 4 split . “Page auto switching time“ automatically switches to another camera after the time(second) you set. |
| (3) | It sets up external monitor printout according to events. “one split popup” prints out on external monitor in case surveillance screen is one split. “Motion popup” prints out on external monitor when motions occur in the system. “Sensor popup” prints out on external monitor when sensor works in the system. “Event dwelling time” sets up event dwelling time(second) to print out on external monitor. |
| (4) | It sets up priority between one-split and event at (3). |

Network setup

(1)

(2)

(3)

(4)

<picture 3.8>

You can set up system network on the screen like <picture 3.8>.

| items | description |
| --- | --- |
| (1) | “Network use” sets up whether to use network on the system with or not. “IP configuration” : inputs IP information after moving to IP setup screen.. “port setup” : sets up network port system uses. Please, inquire network manager to use port. |
| (2) | It sets up RMS to notify when events occur. Use “Add” button to add RMS. |
| (3) | It sets up Email notify when events occur. Recipient “recipient” : Input recipient’s Email address.-“ -“cc” : Input cc’s Email address. – -“Frequency” -“: It notifies you of events through Email and doesn’t notify you of another events occurred during the time you set. “Email Test” : It sends test Email with information set by. -“Default”: -: You can send Emails using the Email server designated by the system. - “SMPT” : Input server address of Email you would send. - “SMPT” : - “From”: Input sender’s Email address. – - “Use login”: Input user’s ID and password to be authorized to use Email server. |
| (4) | It sets up whether to use dynamic IP and dynamic IP. |

Add RMS

<picture 3.8,.1>

Click “ADD” at (2) of <picture 3.8>to add RMS notify through network when events occur.

Click “ADD” after inputting CMS IP and Post at <picture 3.8.1>. Refer to appendix for more details.

Dynamic IP setup

<picture 3.8.2>

To log in from remote places, you have to add system’s dynamic IP at (4) of <picture 3.8>.

You can use it after adding with host’s name and user ID, password in <picture 3.8.2>.

Refer to appendix for more details.

User setup

(1)

(2)

(3)

<picture 3.9>

You can set up system’s user on the screen like <picture 3.9>.

| items | description |
| --- | --- |
| (1) | It prints out added user’s list. “ADD” adds new users. “Delete” deletes added users. “Modify” modifies the information of added users through (2),(3).. |
| (2) | You can modify the information of added users. |
| (3) | You can modify user’s authority and available cameras. |

ADD USER

(1)

(2)

(3)

<picture 3.9.1>

You can add new users using “ADD” at (1) of <picture 3.9>on the screen <picture 3.9.1>.

| items | description |
| --- | --- |
| (1) | Input user ID and password. |
| (2) | Set up new user’s authority. |
| (3) | Set up cameras that new users can use from remote places. |

DVR Search

Screen description

(6)

(2)

(5)

(9)

(8)

(3)

(7)

(4)

(1)

- <picture 4.1>

| Items | description |
| --- | --- |
| (1) | Image screen (upper right on the screen : camera’s number and name. (center bottom on the screen : searching time) |
| (2) | Calendar: Select a day and time to search. |
| (3) | Select cameras(1-4 Cameras : HD, 8~16 Cameras: analog) and time( displays 24hours) |
| (4) | It supports split screen(default split screen displays 4split) and panorama search and full screen.. |
| (5) | Buttons for play and sound. |
| (6) | It support save, smart search, print, color function. |
| (7) | Bookmark and bookmark list |
| (8) | Select data’s image |
| (9) | End |

How to play image

<picture 4.2>

- Click the date you want ( It appears yellow when it has data) Select cameras and time( It changes by minutes when double clicking time bar)

- Default split screen displays 4split and you can select any split screen you want to see.

- Click play button..

- Click audio button to listen to the sound during search.

- <In case of sound saving, data part of time bar appears darker color which is similar with original color.

Split screen

Split screen : the function you use to search several cameras at the same time.

1split

4 splits ( basic split screen of search)

9 S      9 splits

16 Splits

Full screen

Panorama search

This is a function to search images of one channel by a frame in order.

<picture 4.4>

- Select date and camera and time(minute).

- Set up a screen you want to see as one split screen. <Double click screens you want to see or click one split button>

- Click panorama button.

- Click play button.

Image enlargement

This is a function that enlarges images that you want to search and help you search with enlarged images.

<Before enlarging images>

<After enlarging images>

- Click zoominout button

- Enlarge images using the bar on the enlarged screen.

- Move the square with red line on the enlarged using a mouse.

- Put the square on the place you want to search and search images clicking play button.

Bookmark

This is a function that you can search the data you searched once without searching them again.

- It is same with basic search method

- Click stop button on the page you want to put a bookmark during the search.

- Click ADD button at (7) of <picture 4.7> .(

- Input short description on the image you want to put a bookmark and click OK button.

- Check the image you put a bookmark from the book mark list.

Click the images you put bookmarks from the bookmark list and search button, you can search and play the images.

<bookmark>                          <bookmark list>

Image save

“Save” at (6) of <picture 4.1> supports JPEG save, AVI save, CD burn

JPEG save

(1)

(2)

<picture 4.7.1>

It saves the images on the screen as JPEG file.

Select a drive to save images at (1) of <picture 4.7.1> and input file name at (2) and you can save images clicking “OK”.

AVI Save

This is a function that converts images into video file in AVI and able to see the images easily through another PC.

(1)

(2)

(3)

(4)

(5)

<picture 4.7.1>

<picture 4.7.1>

You can back up images as AVI after choosing channel from <picture 4.7.1> .

| items | description |
| --- | --- |
| (1) | Select a camera you want to create as AVI( you can select several cameras). |
| (2) | Input AVI start time and end time. |
| (3) | Select a device to save AVI information. Backed up information is saved under “C:\_Backup” and in case of inputting “Folder name”, “C:\_Backup\folder name” is created and saved. “ |
| (4) | It displays the situation of AVI creating. |
| (5) | Create AVI using “Record start” button. |

CD burn

(1)

(2)

(3)

<picture 4.7.3>

Check a device at (1) and Add folder or file to save at (2) and Create CD/DVD at (3) of <picture 4.7.3>.

Smart search

Smart search of (6) of <picture 4.1> is a function that catches image changes from particular part of the images and prints out them. It able users search images easily with that information

(2)

(1)

(3)

(4)

(5)

<picture 4.8>

| items | description |
| --- | --- |
| (1) | It prints out the screen you would smart search. |
| (2) | Set the sensing area from the (1) image. (1) Click “Set” button and set sensing area dragging a mouse on (1) screen. Delete the selected area with “Delete” button. “Sensitivity” adjusts a sensitivity of selected sensing area and the lower the number is, the more sensible it is. |
| (3) | Input a camera number and sensing start time and end time to sense and start sensing with “search” button. “Calling start image” prints out start image at (1) when you change camera number or start time. |
| (4) | If you click “Search” button of (3), image list that sensed motion in the sensing area set by at (2) prints out on the screen of (1). ( “Add bookmark” adds the images printed out. “Delete list” deletes the list printed out. |
| (5) | These are play buttons for (1) screen. It supports print, save. |

Image print

It prints out searched images through printers.

<picture 4.9>

- It is same with the basic search method.

- Stop playing on images you want to print out.

- Click image print button.

- Input description to the images you want to print and click OK button at <picture 4.9>

- Check the images you would print at <picture 4.9> and print clicking print button.

Image color adjustment

You can adjust the colors of the printed images with “Color” button at (6) of <picture 4.1>.

<picture 4.10)

| items | description |
| --- | --- |
| Original |  |
| Brightness |  |
| Contrast |  |
| Saturation |  |
| Sharpness |  |
| Blur |  |
| Black&White |  |

Selection for data image

It’s the function which recovers and searches the backed up information, supports remote search at <picture 4.1(8)

Appendix 1 PTZ Use

Preparation

| Items | HDS-1204 | HDS-2408 | SDH-H2408 | SDH-H4816 |
| --- | --- | --- | --- | --- |
| DVR Board Image |  |  |  |  |
| PTZ Board name | PTZ |  | EIO0404 | EIO1604 |
| PTZ image |  |  |  |  |
| Cable |  |  |  |  |
| USB Driver |  |  |  |  |
| Etc | Hybrid Model(HYH-4812,HYH-2412) use EIO0404 and EIO1604. |  |  |  |

How to set up H/W

- How to set up PTZ board

- Connect PTZ board to the system.

- Connect 10 box cables to PC mother board’s USB port and PTZ board.

- Connect PTZ power input cable to PC power.

- Install USB driver after turning on the power.

- How to set up EIO PTZ

- Connect PTZ board to the system.

- Connect 20 box cables to DVR board and EIB board.

ㅔ Mother보드 USB Port

Connect PC USB port to EIO USB Port.

- Install USB driver after turning on the system power.

- USB driver installation

Install USB driver with auto install CD.

- Setup of PTZ in DVR

- Go to “Setup” on DVR surveillance page and select “camera” tab.

- Select installed PTZ camera and click “Use” and set up value to fit presently installed PTZ camera.

- Address : address which is set on PTZ camera(receiver’s address):

- Type : Select protocols by companies :

- Port : Serial port to send PTZ data (COM, COM2, COM3…) :

<Note> Set after checking “Port(COM and LPT)” of device manager.

- For communication speed, parity, data bit, stop bit, refer to PTZ camera communication spec.

- PTZ Camera operation

P

- Select a camera that you want to use PTZ.

PTZ control board appears when you click “Pantilt” button on the right.

(1)

(2)

(3)

(4)

(5)

| Items | Description |
| --- | --- |
| (1) | “Power” : PTZ Power ON/OFF “Lamp” : Lamp power ON/OFF “AUX” : Auxiliary device output ON/OFF |
| (2) | Camera’s up and down, left and right, diagonal line, auto pan control |
| (3) | Camera’s number chosen present PTZ |
| (4) | Camera’s zoom, focus, iris control |
| (5) | P/T/Z speed control Installation and operation of Preset Installation and operation of touring Installation and operation of scan |

- Preset setup

What is Preset? It is a function that user previously set up location to receiver and can make the camera surveil the location when user call the information.

(2)

(1)

- Click the preset setup icon button of (1).

- Click “ADD” of (2) on preset setup screen.

- Click “Add” button after inputting preset number and location information.

- Select “preset number” to operate from preset list and Click “Go To” button and “Save” after testing and finish it.

- Select list you want using preset list button and click        button.

<Note> Please note that when you input the repeated preset number, the location information you previously input would be removed and newly input information would be saved.

- Touring setup

What is touring?

This is a function that user make groups using previous preset information and operates it repeatedly and surveil.

(2)

(1)

- Click the touring setup button of (1).

- Add “touring” clicking “ADD” button of (2)

- Select “preset” to operate “touring” from preset list.

- Set up time the camera would stay and click “>>” button and make members.

- Input touring number and member’s name after setting up touring member and click “Add” button.

- Added touring number and member’s name registered on the touring list would be added.

- Touring operates when clicking touring number.

- Click “end” button after “Save”.

- Select list you want using touring list button and click     button and it operates.

Ex) It stops as long as camera stays at 1 location and moves to 2 location

It repeatedly watched out the location set by.

Appendix 2 Sensor/ Relay setup

Preparation

| Items | HDS-1204 | HDS-2408 | SDH-H2408 | SDH-H4816 |
| --- | --- | --- | --- | --- |
| DVR Board image |  |  |  |  |
| PTZ board name | PTZ |  | EIO0404 | EIO1604 |
| PTZ image |  |  |  |  |
| Cable |  |  |  |  |
| Number of Sensor / Relay | 4/4 | 8/8 | 4/4 | 16/4 |
| Etc | Hybrid model(HYH-4812,HYH-2412) use EIO0404, EIO1604. |  |  |  |

Hardware connection

- HDS1204, HDS2408

- Connect box cable to 10 pin box connectors of HD and DIO board.  Connect FDD connection cable of Power to DIO board.

- How to connect HDS1204, HDS2408 sensor/ relay

- SDH-H4816, SDH-H2408

- Connect 20 pin box cables to H.264 board and EIO.

- How to connect SDH-H4816, SDH-H2408 sensor/ relay

No<Note> Default of system is relay type.

|  | Relay Type | Voltage Type |
| --- | --- | --- |
| EIO0404 |  |  |
| EIO1604 |  |  |

- How to set up sensor in DVR

- Go to “Setup” on DVR surveillance page and select “sensor” tab.

- Select installed PTZ camera and click “Use” and set up value to fit presently installed PTZ camera.

(2)

(1)

(3)

(4)

(5)

| Items | Description |
| --- | --- |
| (1) | Select “Sensor”. |
| (2) | Select “Sensor” name and sensor type. |
| (3) | Select how to notify when sensor works. |
| (4) | Select camera to be connected to the sensor. |
| (5) | Select relay to be connected to the sensor. |

- Basic setup of sensor setup is 1~4.

- “Event notify” setup in DVR

It is setup for how to notify DVR user and network clients when events occur.

“Event notify” of picture (3) above notifies DVR user when event occurs.

| Items | Description |
| --- | --- |
| Full screen | Events occur Displays with full screen If you click screen or it reaches duration time, original screen appears again. |
| Beep | Event occur이벤트발생 It beeps If you click screen or it reaches duration time, it stops beeping. |
| Screen display | Event occur이벤트발생 Screen display If you click screen or it reaches duration time, original screen appears again. |

비프

화면 표시

- Network client notify setup

This is a function that notifies network client of events that happen in DVR.

It notifies event occurrence to remote places by CMS, Email like picture (3) above.

- Network client notify schedule setup

It sets up schedule to notify events which occurs in DVR to network client. Go to “Notify” of picture 3 above and click “Schedule setup”

(3)

(1)

(2)

(4)

| Items | Description |
| --- | --- |
| (1) | Select a type of event. “Motion”: Set up schedule to notify when motion occurs. “Sensor”: Set up schedule to notify when sensor works. |
| (2) | Select a camera you apply “notify schedule” to. |
| (3) | Set up schedule to notify to RMS. Click “No operation”(purple)“ and day/time box below and set up. Click “Operation”(green) and day/time box below and set up. |
| (4) | Set up schedule to notify an Email. Click “No operation”(purple)“ and day/time box below and set up. Click “Operation”(green) and day/time box below and set up. |

- Add sites to notify (remote client)

Input “setup” sites you want to add at “Network notify” on “Network tab”. .

(1)

| Items | Description |
| --- | --- |
| (1) | Add sites using “Add” button. Input site address to “CMS IP Address”. Input port address to “CMS Port Number”. <Note> You can add 5 sites in notify site. |

- Set up value of transmission and reception for “Email notify”

(2)

Send by using default transmission server

(1)

(3)

| Items | Description |
| --- | --- |
| (1) | Input receiver’s Email address. |
| (2) | Click “Default transmission server use”. |
| (3) | Set up “Email notify cycle” |

(1)

When you use another Email server without using default server

(2)

| Items | Description |
| --- | --- |
| (1) | Input receiver’s Email address. Clear “Default transmission server use” |
| (2) | “SMTP” : Input Email transmission server. “Port” : Input Email server port. “Sender’s address” : Input “Sender’s address” “Login use” : Input user ID and password when you need to log into main server. |

- CMS setup regarding event

Setup order

- DVR setup (refer to upper page)

- Set up “Level setup” at CMS

- Set up “Scenario setup” at CMS

Event alarm level setup

(1)

(2)

(4)

(3)

| 항목 | 설명 |
| --- | --- |
| (1) | Divide event scenarios by four levels and give scenarios according to importance of events. |
| (2) | Set up basic alarm sound program to output when event occurs. |
| (3) | Set up event duration. |
| (4) | Set up “Email notify” according to event level when event occurs. |

Event scenario setup

Give alarm level to events that occurs on DVR.

(2)

(1)

(3)

(4)

| Items | Description |
| --- | --- |
| (1) | Select site’s motion event and sensor event. |
| (2) | Select event level. |
| (3) | Input messege to output when event occurs. |
| (4) | Input warning voice file to output when event occurs. |

Appendix 3 How to set up TV-Out

Support specs by models

하드웨어 연결 방법

How to connect hardware

HDMI Monitor

Analog Monitor

How to set up TV Out in DVR

Go to “Screen” tab of DVR setup and Click “External Monitor setup”.

(2)

(1)

(3)

(4)

| Items | Description |
| --- | --- |
| (1) | Select camera to output on external monitor. User can select one or several cameras. Selected camera appears as green. |
| (2) | “Split” : sets up external monitor output as 1 split or 4 splits. “Auto switching time”: It automatically switches to another camera after set time. |
| (3) | It sets up external monitor output according to event. “1 split popup” : appears when surveillance screen is 1 split. “Motion popup” appears on external monitor when motion occurs in system. “Sensor popup” appears on external monitor when sensor works in system. “Event duration time: sets up screen duration time which outputs on external monitor. |
| (4) | It sets up priority between 1 split and event at (3) item. |